## Course: Digital Image Processing (Theory)

General Instructions for Students:

- Use A4 size white pages only.

- Solve each question step by step.

- Do not skip any steps; partial steps will not be awarded marks.

- Do not use direct or shortcut methods.

- All mathematical workings must be clearly shown.

- Write neatly and keep your work clean and well-organized.

- Submit your work as a scanned PDF file only.

- Upload the PDF on the LMS before the deadline.

- Treat each given image as a 2D matrix of gray‑level intensities.

- Assume 8‑bit images unless stated otherwise (gray levels 0–255).

- Show all steps for calculations. [CLO-3]

## Question 1: Image Negative Transformation

Given the following 4×4 grayscale image matrix F:

F = [ 25 60 120 200

40 80 140 220

10 90 160 240

0 30 70 255 ]

- Write the mathematical formula for image negative transformation.

- Compute the resulting image matrix G after applying the negative transformation.

- Explain how this transformation affects visual appearance.

## Question 2: Linear Contrast Stretching

Consider the following 3×3 low‑contrast image F:

F = [ 90 95 100

92 97 102

94 96 98 ]

- Identify r_min and r_max of the image.

- Apply linear contrast stretching to map the range [r_min, r_max] → [0, 255].

- Compute the transformed image matrix G.

- Comment on why contrast stretching is useful.

## Question 3: Logarithmic Transformation

Given a 3×3 image matrix F:

F = [ 1 5 10

20 40 80

120 160 200 ]

- Write the log transformation equation.

- Using c = 1, compute the transformed matrix G = log(1 + F).

- Explain which intensity range (low or high) is emphasized by log transformation.

## Question 4: Power‑Law (Gamma) Transformation

Given the following normalized image matrix F (values between 0 and 1):

F = [ 0.10 0.30 0.50

0.60 0.75 0.90

0.20 0.40 0.80 ]

- Apply power‑law transformation with γ = 0.5 and c = 1.

- Compute the resulting matrix G.

- State whether dark or bright pixels are enhanced and why.

## Question 5: Bit‑Plane Slicing

Consider the following 3×3 image F:

F = [ 128 130 135

140 150 160

170 180 200 ]

- Convert each pixel value into 8‑bit binary.

- Extract the 7th and 8th bit‑planes.

- Represent each extracted bit‑plane as a binary matrix.

- Explain the importance of higher‑order bit‑planes.

## Question 6: Spatial Filtering Using Averaging Mask

Given the following 5×5 image F:

F = [ 10 20 30 40 50

20 30 40 50 60

30 40 50 60 70

40 50 60 70 80

50 60 70 80 90 ]

And a 3×3 averaging filter:

H = (1/9) × [ 1 1 1

1 1 1

1 1 1 ]

- Compute the filtered value at the center pixel (3,3) only.

- Show all convolution steps.

- Explain the effect of averaging filters on images.

## Question 7: Geometric Transformation

Given four pixels (0,0), (2,0), (0,2), (2,2) in an image:

- Apply scaling transformation with factors Sx = 2, Sy = 0.5.

- Apply Translation transformation with factors dx = 2, dy = 2.

- Apply shear transformation with factors x = 2, y = 1.

- Find the new pixel coordinates.

- Draw coordinates before and after transformation.

## Question 8: Probabilistic Methods Using Gray-Level Distribution

Consider the following 5×5 grayscale image represented as a 2D matrix F. Pixel intensity levels lie in the range 0–7.

F = [ 0 1 2 3 4

1 2 3 4 5

2 3 4 5 6

3 4 5 6 7

4 5 6 7 7 ]

(a) Probability Mass Function (PMF)

- List all possible gray levels present in the image.

- Count the number of occurrences of each gray level.

- Compute the probability p(rₖ) of each gray level.

(b) Mean and Variance of Gray Levels

- Using the PMF, compute the mean (expected value) of the image intensity.

- Compute the variance of the gray levels.

(c) Cumulative Distribution Function (CDF)

- Compute the cumulative distribution function (CDF) for each gray level.

- Explain how the CDF is used in histogram equalization.